Memory (I/O) address assignment

Each memory chip covers a section of memory address

By analyzing the addresses for a single chip, we find:

- Several most significant bits are in common.

- Several least significant bits various from zeros to ones.

  - Example: 4K * 8-bit chip starting at 10000h

From 10000h to 10FFF

0001 0000 0000 0000 0000

0001 0000 1111 1111 1111

  - What if we have a chip containing 4000 bytes?

Then, no such easy break up...

We prefer memory chip sizes like 256,512,1024…

- The higher part is for CS logic, while the lower part is connected to the chip directly.

Difference between 8088 and 8086

A1

A1

- What is the supported memory size of 8088? And 8086?

- 8086 is fully compatible with 8088.

  - All programs for 8088 must work for 8086.

  - Byte-based memory accesses are the same.

  - Word-based memory accesses are partially optimized.

    - Word alignment matters. See the example:

A word at address A0000h and A0001h

…0000     …0001

…0001     …0010

    - Only even-addressed words will be loaded in one cycle in 8086 processors.

- Data Bus Interface for 8086

  - Even addressed bytes are accessed on D7~D0

  - Odd addressed bytes are accessed on D15~D8

  - Thus, even addressed words is easily on D15~D0

    - A19~A1 represents the word

    - A19~A1 will be sent to both even and odd banks.

  - However, odd-addressed words can not be accessed in one cycle

    - Because the even and odd banks needs different values for A19~A1

    - Such accesses will be two byte-operations.

- How to support this structure?

| A0 = 0 | Byte | Even |
| --- | --- | --- |
|  | Word | Even & Odd |
| A0 = 1 | Byte | Odd |
|  | Word | None |

  - A0 decides the even bank

  - What for the odd bank?  We have /BHE!!!

| BHE* | A0 | CS* |
| --- | --- | --- |
| 0 | 0 | Choose both odd and even memory bank |
| 0 | 1 | Choose only odd memory bank |
| 1 | 0 | Choose only even memory bank |
| 1 | 1 | None is chosen |