Everything about JMP

Syntax

jmp	target

JMP is to modify CS/IP. There are 6 types of jumps.

The assembler differentiates them by their operands:

| Operands | Examples |
| --- | --- |
| jmp disp8 | jmp 20h (8-bit displacement) |
| jmp disp16 | jmp 1212h(16-bit Displacement) |
| jmp adrs32 | jmp 1200:0000h (Address) |
| jmp mem16 | Jmp target[bx][si](Mem. Addr. Modes) |
| jmp reg16 | Jmp AX (Any Reg. is valid.) |
| jmp mem32 | Jmp dword ptr [bx][si] |

Notes:

- If you use an immediate number, it is a displacement.

- If you use memory addressing modes, execution will jump to the address value saved at the location indicated by the addressing mode in the memory. For example, JMP target[bx][si] will not jump to address target+bx+si, but will jump to the address value (either word or double word) saved at  [target+bx+si] in memory.

- For using memory addressing modes in JMP, if you do not have a WORD or DWORD type variable as part of the memory addressing mode, you MUST explicitly indicate the type of the operand by WORD PTR or DWORD PTR.

  - JMP [BX] is NO GOOD.

  - JMP WORD PTR [BX]    ; Short Jump. IP modified.

  - JMP DWORD PTR [BX] ; Far Jump. CS:IP modified.

- If you use a register, IP will be loaded with the value of that register. Any register is valid, not only BX, BP, SI, DI.

- You can jump to other segments if you use a 32-bit operand.

Conditional Jumps

For 80286 and earlier, ALL Conditional Jumps are near jumps. Only an 8-bit displacement is valid. The jump range is between -128 and 128 bytes.

You may read in detail at:

http://maven.smith.edu/~thiebaut/ArtOfAssembly/CH06/CH06-5.html